SDS 940 Simulator Usage

01-Dec-08

(minor additions 28-Feb-14, 17-Mar-14, 08-Apr-14, 17-Apr-14, 19-Apr-14, 20-Apr-14)

COPYRIGHT NOTICE

The following copyright notice applies to the SIMH source, binary, and documentation:

Original code published in 1993-2008, written by Robert M Supnik

Copyright (c) 1993-2008, Robert M Supnik

Permission is hereby granted, free of charge, to any person obtaining a copy of this software and associated documentation files (the "Software"), to deal in the Software without restriction, including without limitation the rights to use, copy, modify, merge, publish, distribute, sublicense, and/or sell copies of the Software, and to permit persons to whom the Software is furnished to do so, subject to the following conditions:

The above copyright notice and this permission notice shall be included in all copies or substantial portions of the Software.

THE SOFTWARE IS PROVIDED "AS IS", WITHOUT WARRANTY OF ANY KIND, EXPRESS OR IMPLIED, INCLUDING BUT NOT LIMITED TO THE WARRANTIES OF MERCHANTABILITY, FITNESS FOR A PARTICULAR PURPOSE AND NONINFRINGEMENT.  IN NO EVENT SHALL ROBERT M SUPNIK BE LIABLE FOR ANY CLAIM, DAMAGES OR OTHER LIABILITY, WHETHER IN AN ACTION OF CONTRACT, TORT OR OTHERWISE, ARISING FROM, OUT OF OR IN

CONNECTION WITH THE SOFTWARE OR THE USE OR OTHER DEALINGS IN THE  SOFTWARE.

Except as contained in this notice, the name of Robert M Supnik shall not be used in advertising or otherwise to promote the sale, use or other dealings in this Software without prior written authorization from Robert M Supnik.

1	Simulator Files	3

2	SDS 940 Features	3

2.1	CPU	4

2.2	Channels (CHAN)	5

2.3	Console Input (TTI)	6

2.4	Console Output (TTO)	6

2.5	Paper Tape Reader (PTR)	6

2.6	Paper Tape Punch (PTP)	7

2.7	Line Printer (LPT)	7

2.8	Real-Time Clock (RTC)	8

2.9	Terminal Multiplexer (MUX)	8

2.10	Project Genie Drum (DRM)	9

2.11	Rapid Access (Fixed Head) Disk (RAD)	10

2.12	Moving Head Disk (DSK)	10

2.13	Magnetic Tape (MT)	11

3	Symbolic Display and Input	12

This memorandum documents the SDS 940 simulator.

# Simulator Files

sim/		scp.h

sim_console.h

sim_defs.h

sim_fio.h

sim_rev.h

sim_sock.h

sim_tape.h

sim_timer.h

sim_tmxr.h

scp.c

sim_console.c

sim_fio.c

sim_sock.c

sim_tape.c

sim_timer.c

sim_tmxr.c

sim/sds/	sds_defs.h

sds_cpu.c

sds_drm.c

sds_dsk.c

sds_io.c

sds_lp.c

sds_mt.c

sds_mux.c

sds_rad.c

sds_stddev.c

sds_sys.c

# SDS 940 Features

The SDS-940 simulator is configured as follows:

device name(s)	simulates

CPU			SDS-940 CPU with 16KW to 64KW of memory

CHAN			I/O channels

PTR			paper tape reader

PTP			paper tape punch

TTI			console input

TTO			console output

LPT			line printer

RTC			real-time clock

MUX			terminal multiplexor

DRM			Project Genie drum

RAD			fixed head disk

DSK			9164/9165 rapid access (moving head) disk

MT			magnetic tape

Most devices can be disabled or enabled with the SET <dev> DISABLED and SET <dev> ENABLED commands, respectively.

The LOAD command is used to load a line printer carriage-control tape.  The DUMP command is not implemented.

## CPU

The CPU options set the size of main memory and the configuration of peripherals.

SET CPU 16K				set memory size = 16KW

SET CPU 32K				set memory size = 32KW

SET CPU 48K				set memory size = 48KW

SET CPU 64K				set memory size = 64KW

SET CPU GENIE			enable DRM, set terminal mux to GENIE mode

SET CPU SDS				disable DRM, set terminal mux	to SDS mode

If memory size is being reduced, and the memory being truncated contains non-zero data, the simulator asks for confirmation.  Data in the truncated portion of memory is lost.  Initial memory size is 64KW.

CPU registers include the visible state of the processor as well as the control registers for the interrupt system.

name			size		comments

P			14		program counter

A			24		accumulator A

B			24		accumulator B

X			24		index register

OV			1		overflow indicator

EM2			3		memory extension, quadrant 2

EM3			3		memory extension, quadrant 3

RL1			24		user relocation register 1

RL2			24		user relocation register 2

RL4			12		kernel relocation register

NML			1		normal mode flag

USR			1		user mode flag

MONUSR		1		monitor-to-user trap enable

ION			1		interrupt enable

INTDEF		1		interrupt defer

INTREQ		32		interrupt request flags

APIACT		5		highest active API level

APIREQ		5		highest requesting API level

XFRREQ		32		device transfer request flags

BPT			4		breakpoint switches

ALERT			6		outstanding alert number

STOP_INVINS		1		stop on invalid instruction

STOP_INVDEV		1		stop on invalid device number

STOP_INVIOP		1		stop on invalid I/O operation

INDLIM		8		maximum indirect nesting depth

EXULIM		8		maximum execute nesting depth

PCQ[0:63]		14		P prior to last branch or interrupt;

most recent P change first

WRU			8		interrupt character

The CPU can maintain a history of the most recently executed instructions.  This is controlled by the SET CPU HISTORY and SHOW CPU HISTORY commands:

SET CPU HISTORY			clear history buffer

SET CPU HISTORY=0			disable history

SET CPU HISTORY=n			enable history, length = n

SHOW CPU HISTORY			print CPU history

SHOW CPU HISTORY=n		print first n entries of CPU history

The maximum length for the history is 65536 entries, the minimum length is 64 entries. This history records the CPU mode when the instruction was executed (normal, monitor or user). The SET CPU HISTORY command accepts one switch value to optionally suppress recording of instructions in a particular CPU mode:

SET –n CPU HISTORY=n		don’t record if in normal mode

SET –m CPU HISTORY=n		don’t record if in monitor mode

SET –u CPU HISTORY=n		don’t record if in user mode

The SDS 940 simulator implements four types of execution breakpoints, controlled by command line switches:

-e		break if P equals address, unqualified by machine mode

-m		break if P equals address, machine in monitor mode

-n		break if P equals address, machine in normal (SDS 930) mode

-u		break if P equals address, machine in user mode

Breakpoint commands default to –e behavior if no switch is specified.

### Next Command

The Next command is supported to step over a BRM, POP, or SYSPOP instruction by installing temporary breakpoints at P+1 and P+2 (and P+3 in the case of a BRM) and then executing the Go command. The temporary breakpoints are removed should execution be interrupted for any other reason. For other instructions, Next is converted to a Step command.

Like the Step command, the Next command accepts a repeat count as an argument.  Given this code sequence:

1532:	BRM	1220	<-- P

1533:	LDA	4663

1534:	POP	3214

1535:	BRU	4436	<-- non-skip return from POP

1536:	BRM	3260	<-- skip return from POP

1537: ZRO   3455	<-- argument to subroutine

1540: BRU	1600	<-- error return

1541:	STA	4652	<-- normal return

The following behavior would be observed for various Next commands executed when P=1532:

Step		advance to location 1220

Next		advance to location 1533

Next 2		advance to location 1534

Next 3		advance to location 1535 or 1536 depending on whether POP skips on return

With P=1536, the behavior is thus:

Step		advance to location 3260

Next		advance to location 1540 or 1541

Unlike the Step command, a side-effect of using temporary breakpoints is that diversions into interrupt or trap code become invisible.  With a Step command, execution dutifully follows into the interrupt or trap routine.

Next command accepts two mutually exclusive switches to modify its behavior.

The –f switch (“forward”) instructs the Next command to set temporary breakpoint(s) following the current location regardless of instruction type.  This is useful at the bottom of loops or to avoid going off into unrelated code should an interrupt or memory paging trap occur.  In this example:

1532:	BRX	1220	<-- P

1533:	LDA	4663

a normal Next would advance to either location 1220 or 1533 depending upon the value in the X register (it is converted to a Step).  However, Next –f would only advance to location 1533, allowing the loop code at 1220 to be executed as many times as specified by X. Note that if the loop code branches elsewhere, rather than make the BRX test, control will be lost unless caught by another breakpoint.

The –a switch (“atomic”) alters Next behavior to be more like a Step command, but using temporary breakpoints.  That is, it will plant a breakpoint at the effective address of a BRM or BRX instruction to catch any transfer there.  Its primary use is to step through code ignoring interrupts and traps.

The behavior of various alternative forms of Next are illustrated in this table for different opcodes, showing where temporary breakpoints are placed. “Step” means Next is converted to a Step command.  “LDA, etc.” means all non-skipping, non-branching opcodes, including I/O instructions.  “SKx” means all Skip instructions. The behavior of Next with an EXU (execute) instruction is dependent on the instruction that is eventually executed.

| Opcode | Next | Next -a (Atomic) | Next -f (forward) |
| --- | --- | --- | --- |
| Bad op | Step | Step | Step |
| BRI | Step | EA | Step |
| BRM, SBRM | P+1, P+2, P+3 | EA+1, P+1, P+2, P+3 | P+1, P+2, P+3 |
| BRR | Step | EA | Step |
| BRU | Step | EA | Step |
| BRX | Step | EA, P+1 | P+1 |
| EXU | ? | ? | ? |
| HLT | Step | Step | Step |
| LDA, etc | Step | P+1 | P+1 |
| SKx | Step | P+1, P+2 | P+1, P+2 |
| POP | P+1, P+2 | 100+OP, P+1, P+2 | P+1, P+2 |
| SYSPOP | P+1, P+2 | 100+OP, P+1, P+2 | P+1, P+2 |

Note that these temporary breakpoints are CPU-mode insensitive, so there is the potential for conflict if execution in a different CPU mode should encounter an instruction address numerically equal to any of these temporary breakpoints.

## Channels (CHAN)

The SDS 940 has up to eight I/O channels, designated W, Y, C, D, E, F, G, and H.  W, Y, C, and D are time-multiplexed communications channels (TMCC); E, F, G, and H are direct access communications channels (DACC).  Unlike real SDS 940 channels, the simulated channels handle 6b, 12b, and 24b transfers simultaneously.  The association between a device and a channel is displayed by the SHOW <dev> CHAN command:

SHOW LPT CHAN

channel=W

The user can change the association with the SET <dev> CHAN=<chan> command, where <chan> is a channel letter:

SET LPT CHAN=E

SHOW LPT CHAN

channel=E

Each channel has nine registers.  The registers are arrays, with entry [0] for channel W, entry [1] for channel Y, etc.

name			size		comments

UAR[0:7]		6		unit address register

WCR[0:7]		15		word count register

MAR[0:7]		16		memory address register

DCR[0:7]		6		data chaining register

WAR[0:7]		24		word assembly register

CPW[0:7]		2		characters per word

CNT[0:7]		3		character count

MODE[0:7]		12		channel mode (from EOM instruction)

FLAG[0:7]		9		channel flags

The user can display all the registers in a channel with the command:

SHOW CHAN channel-letter

## Console Input (TTI)

The console input (TTI) polls the console keyboard for input.  It implements these registers:

name			size		comments

BUF			6		data buffer

XFR			1		transfer ready flag

POS			32		number of characters input

TIME			24		polling interval

By default, the console input is assigned to channel W.

## Console Output (TTO)

The console output (TTO) writes to the simulator console window.  It implements these registers:

name			size		comments

BUF			6		data buffer

XFR			1		transfer ready flag

POS			32		number of characters output

TIME			24		time from I/O initiation to interrupt

By default, the console output is assigned to channel W.

## Paper Tape Reader (PTR)

The paper tape reader (PTR) reads data from a disk file.  The POS register specifies the number of the next data item to be read.  Thus, by changing POS, the user can backspace or advance the reader.

The paper tape reader implements these registers:

name			size		comments

BUF			6		data buffer

XFR			1		transfer ready flag

SOR			1		start of record flag

CHAN			4		active channel

POS			32		position in the input file

TIME			24		time from I/O initiation to interrupt

STOP_IOE		1		stop on I/O error

The paper-tape reader supports the BOOT command.  BOOT PTR simulates the standard console fill sequence.

Error handling is as follows:

error	        STOP_IOE		processed as

not attached	1		report error and stop

0		out of tape

end of file		1		report error and stop

0		out of tape

OS I/O error	x		report error and stop

By default, the paper tape reader is assigned to channel W.

## Paper Tape Punch (PTP)

The paper tape punch (PTP) writes data to a disk file.  The POS register specifies the number of the next data item to be written.  Thus, by changing POS, the user can backspace or advance the punch.

The paper tape punch implements these registers:

name			size		comments

BUF			6		data buffer

XFR			1		transfer ready flag

LDR			1		punch leader flag

CHAN			4		active channel

POS			32		position in the output file

TIME			24		time from I/O initiation to interrupt

STOP_IOE		1		stop on I/O error

Error handling is as follows:

error	        STOP_IOE		processed as

not attached	1		report error and stop

0		out of tape

OS I/O error	x		report error and stop

By default, the paper tape punch is assigned to channel W.

## Line Printer (LPT)

The line printer (LPT) writes data to a disk file.  The POS register specifies the number of the next data item to be written.  Thus, by changing POS, the user can backspace or advance the printer.

The line printer implements these registers:

name			size		comments

BUF[0:131]		8		data buffer

BPTR			8		buffer pointer

XFR			1		transfer ready flag

ERR			1		error flag

CHAN			4		active channel

CCT[0:131]		8		carriage control tape

CCTP			8		pointer into carriage control tape

CCTL			8		length of carriage control tape

SPCINST		24		spacing instruction

POS			32		position in the output file

CTIME			24		inter-character time

PTIME			24		print time

STIME			24		space time

STOP_IOE		1		stop on I/O error

Error handling is as follows:

error	        STOP_IOE		processed as

not attached	1		report error and stop

0		out of paper

OS I/O error	x		report error and stop

By default, the line printer is assigned to channel W.

## Real-Time Clock (RTC)

The real-time clock (RTC) frequency can be adjusted as follows:

SET RTC 60HZ			set frequency to 60Hz

SET RTC 50HZ			set frequency to 50Hz

The default is 60Hz.

The clock implements these registers:

name			size		comments

PIE			1		interrupt enable

TIME			24		tick interval

The real-time clock autocalibrates; the clock interval is adjusted up or down so that the clock tracks actual elapsed time.

## Terminal Multiplexer (MUX)

The terminal multiplexer provides 32 asynchronous interfaces.  In Genie mode, the interfaces are hard-wired; in SDS mode, they implement modem control.  The multiplexer has two controllers: MUX for the scanner, and MUXL for the individual lines.  The terminal multiplexer performs input and output through Telnet sessions connected to a user-specified port.  The ATTACH command specifies the port to be used:

ATTACH MUX <port>			set up listening port

where port is a decimal number between 1 and 65535 that is not being used for other TCP/IP activities.

Each line (each unit of MUXL) supports one option: UC, when set, causes lower case input characters to be automatically converted to upper case. In addition, each line supports output logging.  The SET MUXLn LOG command enables logging on a line:

SET MUXLn filename		log output of line n to filename

The SET MUXLn NOLOG command disables logging and closes the open log file, if any.

Once MUX is attached and the simulator is running, the multiplexor listens for connections on the specified port.  It assumes that the incoming connections are Telnet connections.  The connections remain open until

disconnected either by the Telnet client, a SET MUX DISCONNECT command, or a DETACH MUX command.

Other special multiplexer commands:

SHOW MUX CONNECTIONS 		show current connections

SHOW MUX STATISTICS 		show statistics for active connections

SET MUXLn DISCONNECT		disconnects the specified line.

The controller (MUX) implements these registers:

name			size		comments

STA[0:31]		6		status, lines 0 to 31

RBUF[0:31]		8		receive buffer, lines 0 to 31

XBUF[0:31]		8		transmit buffer, lines 0 to 31

FLAGS[0:127]	1		line flags, 0 to 3 for line 0,

4 to 7 for line 1, etc

SCAN			7		scanner current flag number

SLCK			1		scanner locked flag

TPS			8		character polls per second

The lines (MUXL) implements these registers:

name			size		comments

TIME[0:31]		24		transmit time, lines 0 to 31

The terminal multiplexor does not support save and restore.  All open connections are lost when the simulator shuts down or MUX is detached.

## Project Genie Drum (DRM)

The Project Genie drum (DRM) implements these registers:

name			size		comments

DA			19		drum address

CA			16		core address

WC			14		word count

PAR			12		cumulative sector parity

RW			1		read/write flag

ERR			1		error flag

STA			2		drum state

FTIME			24		channel program fetch time

XTIME			24		interword transfer time

STOP_IOE		1		stop on I/O error

Error handling is as follows:

error	        STOP_IOE		processed as

not attached	1		report error and stop

0		drum not ready

Drum data files are buffered in memory; therefore, end of file and OS I/O errors cannot occur.  Unlike conventional SDS 940 devices, the Project Genie drum does not use a channel.

## Rapid Access (Fixed Head) Disk (RAD)

The rapid access disk (RAD) implements these registers:

name			size		comments

DA			15		disk address

SA			6		sector word address

BP			1		sector byte pointer

XFR			1		data transfer flag

NOBD			1		inhibit increment across track

ERR			1		error flag

CHAN			4		active channel

PROT			8		write protect switches

TIME			24		interval between halfword transfers

STOP_IOE		1		stop on I/O error

Error handling is as follows:

error	        STOP_IOE		processed as

not attached	1		report error and stop

0		disk not ready

The rapid access disk is buffered in memory; end of file and OS I/O errors cannot occur. If it is assigned to channel W, bootstrap fill from the device is permitted. By default, the rapid access disk is assigned to channel E and bootstrapping is not permitted.

## Moving Head Disk (DSK)

DSK options include the ability to make the drive write enabled or write locked:

SET RAD LOCKED			set write locked

SET RAD WRITEENABLED		set write enabled

The moving head disk implements these registers:

name			size		comments

BUF[0:63]		8		transfer buffer

BPTR			9		buffer pointer

BLNT			9		buffer length

DA			21		disk address

INST			24		disk instruction

XFR			1		data transfer flag

ERR			1		error flag

CHAN			4		active channel

WTIME			24		interval between character transfers

STIME			24		seek interval

STOP_IOE		1		stop on I/O error

Error handling is as follows:

error	        STOP_IOE		processed as

not attached	1		report error and stop

0		disk not ready

end of file		x		assume rest of disk is zero

OS I/O error	x		report error and stop

By default, the moving head disk is assigned to channel F.

## Magnetic Tape (MT)

MT options include the ability to make units write enabled or write locked.

SET MTn LOCKED			set unit n write locked

SET MTn WRITEENABLED		set unit n write enabled

Magnetic tape units can be set to a specific reel capacity in MB, or to unlimited capacity:

SET MTn CAPAC=m			set unit n capacity to m MB (0 = unlimited)

SHOW MTn CAPAC			show unit n capacity in MB

Units can also be set ENABLED or DISABLED.  The magnetic tape controller supports the BOOT command.  BOOT MTn simulates the standard console fill sequence for unit n.

The magnetic tape implements these registers:

name			size		comments

BUF[0:131071]	8		transfer buffer

BPTR			18		buffer pointer

BLNT			18		buffer length

XFR			1		data transfer flag

CHAN			4		active channel

INST			24		magtape instruction

EOF			1		end-of-file flag

GAP			1		inter-record gap flag

SKIP			1		skip data flag

CTIME			24		interval between character transfers

GTIME			24		gap interval

POS[0:7]		32		position, drives 0 to 7

STOP_IOE		1		stop on I/O error

Error handling is as follows:

error			processed as

not attached	tape not ready; if STOP_IOE, stop

end of file		end of tape

OS I/O error	end of tape; if STOP_IOE, stop

By default, the magnetic tape is assigned to channel W.

# Symbolic Display and Input

The SDS 940 simulator implements symbolic display and input.  Display is controlled by command line switches:

-a			display as three SDS internal ASCII characters

-c			display as four packed SDS 6b characters

-m			display instruction mnemonics

Input parsing is controlled by the first character typed in or by command line switches:

' or -a		three packed SDS internal ASCII characters

" or -c		four packed SDS 6b characters

alphabetic		instruction mnemonic

numeric		octal number

Instruction input uses (more or less) standard SDS 940 assembler syntax.  There are ten instruction classes:

class			operands		examples		comments

no operand		none			EIR

POP (prog op)	op,addr{,tag}	POP 66,100

I/O			addr{,tag}		EOM 1266

mem reference	addr{,tag}		LDA 400,2

STA* 300		indirect addr

reg change		op op op...		CLA CLB		opcodes OR

shift			cnt{,tag}		LSH 10

operand ignored	any			NOP

chan command	chan			ALC W

chan test		chan			CAT Y

SYSPOP (prog op)  op,addr{,tag}	BRS 42

SBRM 500

CIN* 400,2

All numbers are octal.  Channel designators can be alphabetic (W, Y, C, D, E, F, G, H) or numeric (0-7).  Tags must be 0-7, with 2 indicating indexing.

In addition, all display and input commands may specify how the address is mapped to main memory via a command line switch:

-n			normal, address is an absolute physical address

-x			monitor, map address through the monitor memory map

-u			user, map address through the user memory map

-v			current, map address through the monitor or user map

depending upon current machine mode

Memory mapping defaults to –n behavior if no switch is specified.