VAX-11/780 Simulator Usage

8-Aug-2021

COPYRIGHT NOTICE

The following copyright notice applies to the SIMH source, binary, and documentation:

Original code published in 1993-2016, written by Robert M Supnik

Copyright (c) 1993-2016, Robert M Supnik

Permission is hereby granted, free of charge, to any person obtaining a copy of this software and associated documentation files (the "Software"), to deal in the Software without restriction, including without limitation the rights to use, copy, modify, merge, publish, distribute, sublicense, and/or sell copies of the Software, and to permit persons to whom the Software is furnished to do so, subject to the following conditions:

The above copyright notice and this permission notice shall be included in all copies or substantial portions of the Software.

THE SOFTWARE IS PROVIDED "AS IS", WITHOUT WARRANTY OF ANY KIND, EXPRESS OR IMPLIED, INCLUDING BUT NOT LIMITED TO THE WARRANTIES OF MERCHANTABILITY, FITNESS FOR A PARTICULAR PURPOSE AND NONINFRINGEMENT.  IN NO EVENT SHALL ROBERT M SUPNIK BE LIABLE FOR ANY CLAIM, DAMAGES OR OTHER LIABILITY, WHETHER IN AN ACTION OF CONTRACT, TORT OR OTHERWISE, ARISING FROM, OUT OF OR IN

CONNECTION WITH THE SOFTWARE OR THE USE OR OTHER DEALINGS IN THE  SOFTWARE.

Except as contained in this notice, the name of Robert M Supnik shall not be used in advertising or otherwise to promote the sale, use or other dealings in this Software without prior written authorization from Robert M Supnik.

1	Simulator Files	3

2	VAX780 Features	4

2.1	CPU and System Devices	5

2.1.1	CPU	5

2.1.2	Translation Buffer (TLB)	7

2.1.3	SBI Controller (SBI)	7

2.1.4	Memory Controllers (MCTL0, MCTL1)	8

2.1.5	Time-Of-Day Clock (TODR)	8

2.1.6	Interval Timer (TMR)	8

2.1.7	Unibus Adapter (UBA)	9

2.1.8	Massbus Adapters (MBA0, MBA1)	10

2.2	I/O Device Addressing	10

2.3	Programmed I/O Devices	11

2.3.1	Console Input (TTI)	11

2.3.2	Console Output (TTO)	11

2.3.3	RX01 Console Floppy Disk (RXC)	11

2.3.4	Line Printer (LPT)	12

2.4	Disks	13

2.4.1	RP04/05/06/07, RM02/03/05/80 Disk Pack Drives (RP)	13

2.4.2	RL11/RL01/RL02 Cartridge Disk (RL)	14

2.4.3	RK611/RK06/RK07 Cartridge Disk (HK)	15

2.4.4	UDA50 MSCP Disk Controllers (RQ, RQB, RQC, RQD)	16

2.5	Tapes	17

2.5.1	TM03/TE16/TU45/TU77 Magnetic Tapes (TU)	17

2.5.2	TS11 Magnetic Tape (TS)	18

2.5.3	TUK50 TMSCP Tape Controller (TQ)	19

2.6	Communications Devices	21

2.6.1	DZ11 Terminal Multiplexer (DZ)	21

2.6.2	DHU11 Terminal Multiplexer (VH)	22

2.6.3	DELUA/DEUNA Unibus Ethernet Controllers (XU, XUB)	23

2.6.4	DMC11/DMR11 Unibus DDCMP Controllers	24

2.7	CR11 Card Reader (CR)	29

3	Symbolic Display and Input	30

This memorandum documents the DEC VAX-11/780 simulator.

# Simulator Files

To compile the VAX-11/780, you must define VM_VAX, VAX780, and USE_INT64 as part of the compilation command line.  To enable extended file support (files greater than 2GB), you must define USE_ADDR64 as part of the command line as well.

sim/		scp.h

sim_console.h

sim_defs.h

sim_disk.h

sim_ether.h

sim_fio.h

sim_rev.h

sim_serial.h

sim_sock.h

sim_tape.h

sim_timer.h

sim_tmxr.h

scp.c

sim_console.c

sim_disk.c

sim_ether.c

sim_fio.c

sim_serial.c

sim_sock.c

sim_tape.c

sim_timer.c

sim_tmxr.c

sim/vax/	vax_defs.h

vax780_defs.h

vax_vmb_exe.h

vax_cis.c

vax_cmode.c

vax_cpu.c

vax_cpu1.c

vax_fpa.c

vax_mmu.c

vax_octa.c

vax_sys.c

vax_syscm.c

vax7x0_mba.c

vax780_mem.c

vax780_sbi.c

vax780_stddev.c

vax780_syslist.c

vax780_uba.c

sim/pdp11/	pdp11_cr_dat.h

pdp11_mscp.h

pdp11_uqssp.h

pdp11_xu.h

pdp11_cr.c

pdp11_dmc.c

pdp11_dz.c

pdp11_hk.c

pdp11_lp.c

pdp11_rl.c

pdp11_rp.c

pdp11_rq.c

pdp11_ry.c

pdp11_tq.c

pdp11_ts.c

pdp11_tu.c

pdp11_vh.c

pdp11_xu.c

Additional files are:

sim/vax/	vmb.exe		standard boot code

# VAX780 Features

The VAX780 simulator is configured as follows:

device name(s)	simulates

CPU			VAX-11/780 CPU

TLB			translation buffer

SBI			system bus controller

MCTL0,MTCL1		memory controllers, MS780C with 4MB memory each,

or MS780E with 8MB-64MB each

UBA			DW780 Unibus adapter

MBA0,MBA1		RH780 Massbus adapters

TODR			time-of-day clock

TMR			interval timer

TTI,TTO		console terminal

RXC			console RX01 floppy disk

DZ			DZ11 8-line terminal multiplexer (up to 32)

VH			DHU11 16-line terminal multiplexer (up to 4)

CR			CR11 card reader

LPT			LP11 line printer

RP			RP04/05/06/07, RM02/03/05/80 Massbus disks, up to

eight

HK			RK611/RK06(7) cartridge disk controller with eight

drives

RL			RL11/RL01(2) cartridge disk controller with four drives

RQ			UDA50 MSCP controller with four drives

RQB			second UDA50 MSCP controller with four drives

RQC			third UDA50 MSCP controller with four drives

RQD			fourth UDA50 MSCP controller with four drives

RY			RX211 floppy disk controller with two drives

TS			TS11 magnetic tape controller with one drive

TQ			TUK50 TMSCP magnetic tape controller with four drives

TU			TM03 tape formatter with eight TE16/TU45/TU77 drives

XU			DEUNA/DELUA Ethernet controller

XUB			second DEUNA/DELUA Ethernet controller

DMC			DMC11/DMR11 Synchronous network controller

The DZ, VH, LPT, RP, RL, RQ, RQB, RQC, RQD, RY, TS, TQ, TU, XU, XUB and DMC devices can be set DISABLED.  RQB, RQC, RQD, VH, XU, XUB and DMC are disabled by default.

The VAX780 simulator implements several unique stop conditions:

- Change mode to interrupt stack

- Illegal vector (bits<1:0> = 2 or 3)

- Unexpected exception during interrupt or exception

- Process PTE in P0 or P1 space instead of system space

- Unknown IPL

- Infinite loop (BRB/W to self at IPL 1F)

The LOAD command supports a simple binary format, consisting of a stream of binary bytes without origin or checksum, for loading memory.  The DUMP command is not implemented.

## CPU and System Devices

### CPU

CPU options include the size of main memory and the treatment of the HALT instruction.

SET CPU 8M				set memory size = 8MB

SET CPU 16M				set memory size = 16MB

SET CPU 32M				set memory size = 32MB

SET CPU 48M				set memory size = 48MB

SET CPU 64M				set memory size = 64MB

SET CPU 128M			set memory size = 128MB

The CPU also implements a command to display a virtual to physical address translation:

SHOW {-kesu} CPU VIRTUAL=n	show translation for address n

in kernel/exec/supervisor/user mode

The CPU also implements the console AUTORESTART switch:

SET CPU CONHALT			enable AUTORESTART

SET CPU SIMHALT			disable AUTORESTART

Notes on memory size:

- The first version of the VAX-11/780 used MS780C controllers, which supported 1-4MB of memory per controller.  This is the only memory controller recognized by VMS V1.  MS780E controllers supported 4MB-64MB per controller.

- The controller type is set automatically based on memory size.

Initial memory size is 8MB.

Memory can be loaded with a binary byte stream using the LOAD command.  The LOAD command recognizes three switches:

-o			origin argument follows file name

-r			load ROM in memory controller 0

-s			load ROM in memory controller 1

These switches are recognized when examining or depositing in CPU memory:

-b			examine/deposit bytes

-w			examine/deposit words

-l			examine/deposit longwords

-d			data radix is decimal

-o			data radix is octal

-h			data radix is hexadecimal

-m			examine (only) VAX instructions

-p			examine/deposit PDP-11 (compatibility mode) instructions

-r			examine (only) RADIX50 encoded data

-v			interpret address as virtual, current mode

-k			interpret address as virtual, kernel mode

-e			interpret address as virtual, executive mode

-s			interpret address as virtual, supervisor mode

-u			interpret address as virtual, user mode

CPU registers include the visible state of the processor as well as the control registers for the interrupt system.

name			size		comments

PC			32		program counter

R0 .. R14		32		R0 to R14

AP			32		alias for R12

FP			32		alias for R13

SP			32		alias for R14

PSL			32		processor status longword

CC			4		condition codes, PSL<3:0>

KSP			32		kernel stack pointer

ESP			32		executive stack pointer

SSP			32		supervisor stack pointer

USP			32		user stack pointer

IS			32		interrupt stack pointer

SCBB			32		system control block base

PCBB			32		process control block base

P0BR			32		P0 base register

P0LR			22		P0 length register

P1BR			32		P1 base register

P1LR			22		P1 length register

SBR			32		system base register

SLR			22		system length register

SISR			16		software interrupt summary register

ASTLVL		4		AST level register

MAPEN			1		memory management enable

PME			1		performance monitor enable

TRPIRQ		8		trap/interrupt pending

CRDERR		1		correctible read data error flag

MEMERR		1		memory error flag

PCQ[0:63]		32		PC prior to last PC change or interrupt;

most recent PC change first

WRU			8		interrupt character

The CPU attempts to detect when the simulator is idle.  When idle, the simulator does not use any resources on the host system.  Idle detection is controlled by the SET IDLE and SET NOIDLE commands:

SET CPU IDLE{=VMS|ULTRIX| ULTRIXOLD|ULTRIX-1.X|

3BSD|4.0BSD|4.1BSD|4.2BSD|QUASIJARUS

NETBSD|NETBSDOLD|OPENBSD|OPENBSDOLD|32V|ELN}{:n}

enable idle detection

SET CPU NOIDLE			disable idle detection

Idle detection is disabled by default.  If idle detection is enabled with an incorrect operating system setting, simulator performance could be impacted.  The default operating system setting is VMS. The value ‘n’, if present in the “SET CPU IDLE={OS}:n” command, indicates the number of seconds the simulator must run before idling starts.

The CPU can maintain a history of the most recently executed instructions.  This is controlled by the SET CPU HISTORY and SHOW CPU HISTORY commands:

SET CPU HISTORY			clear history buffer

SET CPU HISTORY=0			disable history

SET CPU {-T} HISTORY=n{:file}	enable history, length = n

SHOW CPU HISTORY			print CPU history

SHOW CPU HISTORY=n		print first n entries of CPU history

The –T switch causes simulator time to be recorded (and displayed) with each history entry.  When writing history to a file (SET CPU HISTORY=n:file), ‘n’ specifies the buffer flush frequency.  Warning: prodigious amounts of disk space may be consumed.  The maximum length for the history is 250000 entries.

### Translation Buffer (TLB)

The translation buffer consists of two units, representing the system and user translation buffers, respectively.  It has no registers.  Each translation buffer entry consists of two 32b words, as follows:

word n		tag

word n+1		cached PTE

An invalid entry is indicated by a tag of 0xFFFFFFFF.

### SBI Controller (SBI)

The SBI is the VAX-11/780 system bus.  The simulated SBI implements these registers:

name			size		comments

NREQ14		16		Nexus IPL14 interrupt requests

NREQ15		16		Nexus IPL15 interrupt requests

NREQ16		16		Nexus IPL16 interrupt requests

NREQ17		16		Nexus IPL17 interrupt requests

WCSA			16		writeable control store address

WCSD			32		writeable control store data

MBRK			13		microbreak register

SBIFS			32		SBI fault status

SBISC			32		SBI silo compare

SBIMT			32		SBI maintenance register

SBIER			32		SBI error status

SBITMO		32		SBI timeout address

### Memory Controllers (MCTL0, MCTL1)

The memory controllers implement the registers for the MS780C (8MB memory) or MS780E (16MB or greater memory).  Each controller implements these registers:

name			size		comments

CRA			32		control register A

CRB			32		control register B

CRC			32		control register C

CRD			32		control register D (MS780E only)

ROM[0:1023]		32		bootstrap ROM

ROM can be loaded from a file with the commands

LOAD -R <file>			load MCTL0 ROM

LOAD -S <file>			load MCTL1 ROM

### Time-Of-Day Clock (TODR)

The TODR tracks time since an arbitrary start in 10 millisecond intervals.  It has these registers:

name			size		comments

TODR			32		time-of-day register

TIME			24		delay between ticks

The TODR register autocalibrates against real-world time.

There are two modes of TODR operation.

- Default VMS mode.  Without initializing the TODR it returns the current time of year offset which VMS would set the clock to if VMS knew the correct time (i.e. by manual input).  This is correct almost all the time unless a VMS disk hadn’t been booted from in the current year.  This mode produces strange time results for non VMS OSes on each system boot.

- OS Agnostic mode.  This mode behaves precisely like the VAX780 TODR and works correctly for all OSes.  This mode is enabled by attaching the CLK to a battery backup state file for the TOY clock (i.e. sim> attach CLK TOY_CLOCK).  When operating in OS Agnostic mode, the TODR will initially start counting from 0 and be adjusted differently when an OS specifically writes to the TODR.  VMS determines if the TODR currently contains a valid time if the value it sees is less than about 1 month.  If the time isn't valid VMS will prompt to set the time during the system boot.  While prompting for the time it will wait for an answer to the prompt for up to the SYSGEN parameter TIMEPROMPTWAIT seconds.  A value of 0 for TIMEPROMPTWAIT will disable the clock setting prompt.

### Interval Timer (TMR)

The interval timer implements the VAX architectural timer, with 1 microsecond intervals.  It has these registers:

name			size		comments

ICCS			32		interval timer control and status

ICR			32		interval count register

NICR			32		next interval count register

INT			1		interrupt request

For standard VMS intervals (10 milliseconds), the interval timer autocalibrates against real-world time.

### Unibus Adapter (UBA)

The Unibus adapter (UBA) simulates the DW780.  It recognizes these options:

SET UBA AUTOCONFIGURE		enable autoconfiguration

SET UBA NOAUTOCONFIGURE		disable autoconfiguration

and this SHOW command:

SHOW UBA IOSPACE			display IO address space assignments

The UBA also implements a command to display a Unibus address to physical address translation:

SHOW UBA VIRTUAL=n		show translation for Unibus address n

Finally, the UBA implements main memory examination and modification via the Unibus map.  The data width is always 16b:

EX UBA 0/10				examine main memory words corresponding

to Unibus addresses 0-10

The UBA has these registers:

name			size		comments

IPL14			32		Unibus IPL14 interrupt requests

IPL15			32		Unibus IPL15 interrupt requests

IPL16			32		Unibus IPL16 interrupt requests

IPL17			32		Unibus IPL17 interrupt requests

CNFR			32		configuration register

CR			32		control register

SR			32		status register

DR			32		diagnostic register

INT			1		internal UBA interrupt request

NEXINT		1		UBA Nexus interrupt request

AIIP			1		adapter initialization in progress flag

UIIP			1		Unibus initialization in progress flag

FMER			32		failing memory address

FUBAR			32		failing UBA map register

BRSVR0		32		spare register 0

BRSVR1		32		spare register 1

BRSVR2		32		spare register 2

BRSVR3		32		spare register 3

BRRVR4		32		vector register, IPL 14

BRRVR5		32		vector register, IPL 15

BRRVR6		32		vector register, IPL 16

BRRVR7		32		vector register, IPL 17

DPR[0:15]		32		data path registers 0 to 15

MAP[0:495]		32		map registers 0 to 495

AITIME		24		adapter initialization time

UITIME		24		Unibus initialization time

### Massbus Adapters (MBA0, MBA1)

The Massbus adapters (MBA0, MBA1) simulate RH780's.  MBA0 is assigned to the RP disk drives, MBA1 to the TU tape drives.  Each MBA has these registers:

name			size		comments

CNFR			32		configuration register

CR			32		control register

SR			32		status register

VA			17		virtual address register

BC			32		byte count register

DR			32		diagnostic register

SMR			32		selected map register

MAP[0:255]		32		map registers

NEXINT		1		MBA Nexus interrupt request

## I/O Device Addressing

Unibus I/O space is not large enough to allow all possible devices to be configured simultaneously at fixed addresses.  Instead, many devices have floating addresses; that is, the assigned device address depends on the presence of other devices in the configuration:

DZ11			all instances have floating addresses

RL11			first instance has fixed address, rest floating

RX11/RX211		first instance has fixed address, rest floating

DEUNA/DELUA		first instance has fixed address, rest floating

MSCP disk		first instance has fixed address, rest floating

TMSCP tape		first instance has fixed address, rest floating

To maintain addressing consistency as the configuration changes, the simulator implements DEC's standard I/O address and vector autoconfiguration algorithms for all Unibus devices.  This allows the user to enable or disable devices without needing to manage I/O addresses and vectors.

Autoconfiguration cannot solve address conflicts between devices with overlapping fixed addresses.  For example, with default I/O page addressing, the PDP-11 can support either a TM11 or a TS11, but not both, since they use the same I/O addresses.

In addition to autoconfiguration, most devices support the SET <device> ADDRESS command, which allows the I/O page address of the device to be changed, and the SET <device> VECTOR command, which allows the vector of the device to be changed.  Explicitly setting the I/O address or vector of a device DISABLES autoconfiguration for that device and for the entire system.  As a consequence, the user may have to manually configure all other autoconfigured devices, because the autoconfiguration algorithm no longer recognizes the explicitly configured device.  A device can be reset to autoconfigure with the SET <device> AUTOCONFIGURE command.  Autoconfiguration can be restored for the entire system with the SET UBA AUTOCONFIGURE command.

The current I/O map can be displayed with the SHOW UBA IOSPACE command. Addresses that have set by autoconfiguration are marked with an asterisk (*).

All devices support the SHOW <device> ADDRESS and SHOW <device> VECTOR commands, which display the device address and vector, respectively.

## Programmed I/O Devices

### Console Input (TTI)

The terminal interfaces (TTI, TTO) can be set to one of three modes, 7P, 7B or 8B:

mode	input characters		output characters

7P	high-order bit cleared	high-order bit cleared,

non-printing characters suppressed

7B	high-order bit cleared	high-order bit cleared

8B	no changes			no changes

The default mode is 8B.

When the console terminal is attached to a Telnet session, it recognizes BREAK.  If BREAK is entered, and BDR<7> is set, control returns to the console firmware; otherwise, BREAK is treated as a normal terminal input condition.

The terminal input (TTI) polls the console keyboard for input.  It implements these registers:

name			size		comments

BUF			8		last data item processed

CSR			16		control/status register

INT			1		interrupt pending flag

ERR			1		error flag (CSR<15>)

DONE			1		device done flag (CSR<7>)

IE			1		interrupt enable flag (CSR<6>)

POS			32		number of characters input

TIME			24		input polling interval (if 0, the keyboard

is polled synchronously with the TODR)

### Console Output (TTO)

The terminal output (TTO) writes to the simulator console window.  It implements these registers:

name			size		comments

BUF			8		last data item processed

CSR			16		control/status register

INT			1		interrupt pending flag

ERR			1		error flag (CSR<15>)

DONE			1		device done flag (CSR<7>)

IE			1		interrupt enable flag (CSR<6>)

POS			32		number of characters input

TIME			24		time from I/O initiation to interrupt

### RX01 Console Floppy Disk (RXC)

RX01 options include the ability to set units write enabled or write locked:

SET RXCn LOCKED			set unit n write locked

SET RXCn WRITEENABLED		set unit n write enabled

The RX01 implements a special command, FLOAD, for loading VAX executables from an RT11-formatted console floppy disk image:

FLOAD <file_name> {<origin>}

FLOAD searches the floppy disk image attached to the RX01 for the named file and then loads it into VAX-11/780 memory starting at the origin.  If no origin is specified, the default origin is 200 (hex).

The RX01 implements these registers:

name			size		comments

FNC			8		function select

ES			8		error status

ECODE			8		error code

TA			8		track address

SA			8		sector address

STATE			4		protocol state

BPTR			7		data buffer pointer

CTIME			24		command initiation delay

STIME			24		seek time delay, per track

XTIME			24		transfer time delay, per byte

STOP_IOE		1		stop on I/O error

DBUF[0:127]		8		data buffer

Error handling is as follows:

error	        STOP_IOE		processed as

not attached	1		report error and stop

0		disk not ready

RX01 data files are buffered in memory; therefore, end of file and OS I/O errors cannot occur.

### Line Printer (LPT)

The line printer (LPT) writes data to a disk file.  The POS register specifies the number of the next data item to be written.  Thus, by changing POS, the user can backspace or advance the printer.

The line printer implements these registers:

name			size		comments

BUF			8		last data item processed

CSR			16		control/status register

INT			1		interrupt pending flag

ERR			1		error flag (CSR<15>)

DONE			1		device done flag (CSR<7>)

IE			1		interrupt enable flag (CSR<6>)

POS			32		position in the output file

TIME			24		time from I/O initiation to interrupt

STOP_IOE		1		stop on I/O error

Error handling is as follows:

error	        STOP_IOE		processed as

not attached	1		report error and stop

0		out of paper

OS I/O error	x		report error and stop

## Disks

All VAX-11/780 disks, and the TUK50 MSCP tape, support a special form of the boot command, with the following syntax:

BOOT <unit>{/R5:<value>}

For example,

BOOT RP0/R5:1

The optional switch, /R5, specifies that R5 is to be loaded with the specified value prior to booting.  If the switch is omitted, R5 is loaded with 0.

### RP04/05/06/07, RM02/03/05/80 Disk Pack Drives (RP)

The RP controller implements the Massbus family of large disk drives.  RP options include the ability to set units write enabled or write locked, to set the drive type to one of six disk types, or autosize, and to write a DEC standard 144 compliant bad block table on the last track:

SET RPn LOCKED			set unit n write locked

SET RPn WRITEENABLED		set unit n write enabled

SET RPn RM03			set type to RM03

SET RPn RM05			set type to RM05

SET RPn RM80			set type to RM80

SET RPn RP04			set type to RP04

SET RPn RP05			set type to RP05

SET RPn RP06			set type to RP06

SET RPn RP07			set type to RP07

SET RPn AUTOSIZE			set type based on file size at attach

SET RPn BADBLOCK			write bad block table on last track

The type options can be used only when a unit is not attached to a file.  The bad block option can be used only when a unit is attached to a file.  Units can be set ENABLED or DISABLED.  The RP controller supports the BOOT command.

The RP controller implements the registers listed below.  Registers suffixed with [0:7] are replicated per drive.

name			size		comments

CS1[0:7]		16		current operation

DA[0:7]		16		desired surface, sector

DS[0:7]		16		drive status

ER1[0:7]		16		drive errors

OF[0:7]		16		offset

DC[0:7]		16		desired cylinder

ER2[0:7]		16		error status 2

ER3[0:7]		16		error status 3

EC1[0:7]		16		ECC syndrome 1

EC2[0:7]		16		ECC syndrome 2

MR[0:7]		16		maintenance register

MR2[0:7]		16		maintenance register 2 (RM only)

HR[0:7]		16		holding register (RM only)

STIME			24		seek time, per cylinder

RTIME			24		rotational delay

STOP_IOE		1		stop on I/O error

Error handling is as follows:

error	        STOP_IOE		processed as

not attached	1		report error and stop

0		disk not ready

end of file		x		assume rest of disk is zero

OS I/O error	x		report error and stop

### RL11/RL01/RL02 Cartridge Disk (RL)

RL11 options include the ability to set units write enabled or write locked, to set the drive type to RL01, RL02, or autosize, and to write a DEC standard 144 compliant bad block table on the last track:

SET RLn LOCKED			set unit n write locked

SET RLn WRITEENABLED		set unit n write enabled

SET RLn RL01			set type to RL01

SET RLn RL02			set type to RL02

SET RLn AUTOSIZE			set type based on file size at attach

SET RLn BADBLOCK			write bad block table on last track

The type options can be used only when a unit is not attached to a file.  The bad block option can be used only when a unit is attached to a file.  Units can be set ENABLED or DISABLED.  The RL11 supports the BOOT command.

The RL11 implements these registers:

name			size		comments

RLCS			16		control/status

RLDA			16		disk address

RLBA			16		memory address

RLBAE			6		memory address extension (RLV12)

RLMP,RLMP1,RLMP2	16		multipurpose register queue

INT			1		interrupt pending flag

ERR			1		error flag (CSR<15>)

DONE			1		device done flag (CSR<7>)

IE			1		interrupt enable flag (CSR<6>)

STIME			24		seek time, per cylinder

RTIME			24		rotational delay

STOP_IOE		1		stop on I/O error

Error handling is as follows:

error	        STOP_IOE		processed as

not attached	1		report error and stop

0		disk not ready

end of file		x		assume rest of disk is zero

OS I/O error	x		report error and stop

### RK611/RK06/RK07 Cartridge Disk (HK)

RK611 options include the ability to set units write enabled or write locked, to set the drive type to RK06, RK07, or autosize, and to write a DEC standard 144 compliant bad block table on the last track:

SET HKn LOCKED			set unit n write locked

SET HKn WRITEENABLED		set unit n write enabled

SET HKn RK06			set type to RK06

SET HKn RK07			set type to RK07

SET HKn AUTOSIZE			set type based on file size at attach

SET HKn BADBLOCK			write bad block table on last track

The type options can be used only when a unit is not attached to a file.  The bad block option can be used only when a unit is attached to a file.  Units can be set ENABLED or DISABLED.  The RK611 supports the BOOT command.

The RK611 implements these registers:

name			size		comments

HKCS1			16		control/status 1

HKWC			16		word count

HKBA			16		bus address

HKDA			16		desired surface, sector

HKCS2			16		control/status 2

HKDS[0:7]		16		drive status, drives 0 to 7

HKER[0:7]		16		drive errors, drives 0 to 7

HKDB[0:2]		16		data buffer silo

HKDC			16		desired cylinder

HKOF			8		offset

HKMR			16		maintenance register

HKSPR			16		spare register

HKCI			1		controller interrupt flop

HKDI			1		drive interrupt flop

HKEI			1		error interrupt flop

INT			1		interrupt pending flag

ERR			1		error flag (CSR<15>)

DONE			1		device done flag (CSR1<7>)

IE			1		interrupt enable flag (CSR1<6>)

STIME			24		seek time, per cylinder

RTIME			24		rotational delay

MIN2TIME		24		minimum time between DONE and ATA

STOP_IOE		1		stop on I/O error

Error handling is as follows:

error	        STOP_IOE		processed as

not attached	1		report error and stop

0		disk not ready

end of file		x		assume rest of disk is zero

OS I/O error	x		report error and stop

### UDA50 MSCP Disk Controllers (RQ, RQB, RQC, RQD)

The simulator implements four MSCP disk controllers, RQ, RQB, RQC, RQD.  Initially, RQB, RQC, and RQD are disabled.  Each RQ controller simulates an UDA50 MSCP disk controller with four drives.  RQ options include the ability to set units write enabled or write locked, and to set the drive type to one of many disk types:

SET RQn LOCKED			set unit n write locked

SET RQn WRITEENABLED		set unit n write enabled

SET RQn RX50			set type to RX50

SET RQn RX33			set type to RX33

SET RQn RD51			set type to RD51

SET RQn RD52			set type to RD52

SET RQn RD53			set type to RD53

SET RQn RD54			set type to RD54

SET RQn RD31			set type to RD31

SET RQn RA81			set type to RA81

SET RQn RA82			set type to RA82

set RQn RA71			set type to RA71

SET RQn RA72			set type to RA72

SET RQn RA90			set type to RA90

SET RQn RA92			set type to RA92

SET RQn RRD40			set type to RRD40 (CD ROM)

SET RQn RAUSER{=n}		set type to RA82 with n MB's

SET -L RQn RAUSER{=n}		set type to RA82 with n LBN's

The type options can be used only when a unit is not attached to a file.  RAUSER is a "user specified" disk; the user can specify the size of the disk in either MB (1000000 bytes) or logical block numbers (LBN's, 512 bytes each).  The minimum size is 5MB; the maximum size is 2GB without extended file support, 1TB with extended file support.

Units can be set ENABLED or DISABLED.  The RQ controllers support the BOOT command.

Drive units have changeable unit numbers.  Unit numbers can be changed with:

SET RQn UNIT=val			set unit plug value

Each device has 4 units which have unique MSCP unit numbers (0, 1, 2 and 3).

Each RQ controller implements the following special SHOW commands:

SHOW RQn TYPE			show drive type

SHOW RQ RINGS			show command and response rings

SHOW RQ FREEQ			show packet free queue

SHOW RQ RESPQ			show packet response queue

SHOW RQ UNITQ			show unit queues

SHOW RQ ALL				show all ring and queue state

SHOW RQn UNITQ			show unit queues for unit n

SHOW RQn UNIT			show unit plug value

Each RQ controller implements these registers:

name			size		comments

SA			16		status/address register

S1DAT			16		step 1 init host data

CQBA			22		command queue base address

CQLNT			8		command queue length

CQIDX			8		command queue index

RQBA			22		request queue base address

RQLNT			8		request queue length

RQIDX			8		request queue index

FREE			5		head of free packet list

RESP			5		head of response packet list

PBSY			5		number of busy packets

CFLGS			16		controller flags

CSTA			4		controller state

PERR			9		port error number

CRED			5		host credits

HAT			17		host available timer

HTMO			17		host timeout value

CPKT[0:3]		5		current packet, units 0 to 3

PKTQ[0:3]		5		packet queue, units 0 to 3

UFLG[0:3]		16		unit flags, units 0 to 3

PLUG[0:3]		16		unit plug values, units 0 to 3

INT			1		interrupt request

ITIME			1		response time for initialization steps

(except for step 4)

QTIME			24		response time for 'immediate' packets

XTIME			24		response time for data transfers

PKTS[33*32]		16		packet buffers, 33W each, 32 entries

While VMS is not timing sensitive, most of the BSD-derived operating systems (NetBSD, OpenBSD, etc) are.  The QTIME and XTIME parameters are set to values that allow these operating systems to run correctly.

Error handling is as follows:

error	     		processed as

not attached	disk not ready

end of file		assume rest of disk is zero

OS I/O error	report error and stop

## Tapes

### TM03/TE16/TU45/TU77 Magnetic Tapes (TU)

The TU controller implements the Massbus family of 800/1600bpi magnetic tape drives.  TU options include the ability to set the drive type to one of three drives (TE16, TU45, or TU77), and to set the drives write enabled or write locked.

SET TUn TE16			set unit n drive type to TE16

SET TUn TU45			set unit n drive type to TU45

SET TUn TU77			set unit n drive type to TU77

SET Tun LOCKED			set unit n write locked

SET Tun WRITEENABLED		set unit n write enabled

Magnetic tape units can be set to a specific reel capacity in MB, or to unlimited capacity:

SET TUn CAPAC=m			set unit n capacity to m MB (0 = unlimited)

SHOW TUn CAPAC			show unit n capacity in MB

Units can be set ENABLED or DISABLED.  The TU controller does not support the BOOT command.

The TU controller implements the following registers:

name			size		comments

CS1			6		current operation

FC			16		frame count

FS			16		formatter status

ER			16		formatter errors

CC			16		check character

MR			16		maintenance register

TC			16		tape control register

TIME			24		operation execution time

UST			17		unit status, drives 0 to 7

POS			32		position, drive 0 to 7

STOP_IOE		1		stop of I/O error

Error handling is as follows:

error			processed as

not attached	tape not ready; if STOP_IOE, stop

end of file		bad tape

OS I/O error	parity error; if STOP_IOE, stop

### TS11 Magnetic Tape (TS)

TS options include the ability to make the unit write enabled or write locked.

SET TS LOCKED			set unit write locked

SET TS WRITEENABLED		set unit write enabled

The TS drive can be set to a specific reel capacity in MB, or to unlimited capacity:

SET TS0 CAPAC=m			set capacity to m MB (0 = unlimited)

SHOW TS0 CAPAC			show capacity in MB

The TS11 does not support the BOOT command.

The TS controller implements these registers:

name			size		comments

TSSR			16		status register

TSBA			16		bus address register

TSDBX			16		data buffer extension register

CHDR			16		command packet header

CADL			16		command packet low address or count

CADH			16		command packet high address

CLNT			16		command packet length

MHDR			16		message packet header

MRFC			16		message packet residual frame count

MXS0			16		message packet extended status 0

MXS1			16		message packet extended status 1

MXS2			16		message packet extended status 2

MXS3			16		message packet extended status 3

MXS4			16		message packet extended status 4

WADL			16		write char packet low address

WADH			16		write char packet high address

WLNT			16		write char packet length

WOPT			16		write char packet options

WXOPT			16		write char packet extended options

ATTN			1		attention message pending

BOOT			1		boot request pending

OWNC			1		if set, tape owns command buffer

OWNM			1		if set, tape owns message buffer

TIME			24		delay

POS			32		position

Error handling is as follows:

error			processed as

not attached	tape not ready

end of file		bad tape

OS I/O error	fatal tape error

### TUK50 TMSCP Tape Controller (TQ)

The TQ controller simulates the TUK50 TMSCP tape controller.  TQ options include the ability to set units write enabled or write locked, and to specify the controller type and tape length:

SET TQn LOCKED			set unit n write locked

SET TQn WRITEENABLED		set unit n write enabled

SET TQ TK50				set controller type to TK50

SET TQ TK70				set controller type to TK70

SET TQ TU81				set controller type to TU81

SET TQ TKUSER{=n}			set controller type to TK50 with tape

capacity of n MB

User-specified capacity must be between 50 and 2000 MB.  The TUK50 supports the BOOT command.

Regardless of the controller type, individual units can be set to a specific reel capacity in MB, or to unlimited capacity:

SET TQn CAPAC=m			set unit n capacity to m MB (0 = unlimited)

SHOW TQn CAPAC			show unit n capacity in MB

Drive units have changeable unit numbers.  Unit numbers can be changed with:

SET TQn UNIT=val			set unit plug value

Device TQ has 4 units (TQ0, TQ1, TQ2 and TQ3) which have unique MSCP unit numbers (0, 1, 2 and 3).

The TQ controller implements the following special SHOW commands:

SHOW TQ TYPE			show controller type

SHOW TQ RINGS			show command and response rings

SHOW TQ FREEQ			show packet free queue

SHOW TQ RESPQ			show packet response queue

SHOW TQ UNITQ			show unit queues

SHOW TQ ALL				show all ring and queue state

SHOW TQn UNITQ			show unit queues for unit n

SHOW TQn UNIT			show unit plug value

The TQ controller implements these registers:

name			size		comments

SA			16		status/address register

S1DAT			16		step 1 init host data

CQBA			22		command queue base address

CQLNT			8		command queue length

CQIDX			8		command queue index

RQBA			22		request queue base address

RQLNT			8		request queue length

RQIDX			8		request queue index

FREE			5		head of free packet list

RESP			5		head of response packet list

PBSY			5		number of busy packets

CFLGS			16		controller flags

CSTA			4		controller state

PERR			9		port error number

CRED			5		host credits

HAT			17		host available timer

HTMO			17		host timeout value

CPKT[0:3]		5		current packet, units 0 to 3

PKTQ[0:3]		5		packet queue, units 0 to 3

UFLG[0:3]		16		unit flags, units 0 to 3

PLUG[0:3]		16		unit plug values, units 0 to 3

POS[0:3]		32		tape position, units 0 to 3

OBJP[0:3]		32		object position, units 0 to 3

INT			1		interrupt request

ITIME			1		response time for initialization steps

(except for step 4)

QTIME			24		response time for 'immediate' packets

XTIME			24		response time for data transfers

PKTS[33*32]		16		packet buffers, 33W each, 32 entries

Error handling is as follows:

error	     		processed as

not attached	tape not ready

end of file		end of medium

OS I/O error	fatal tape error

## Communications Devices

### DZ11 Terminal Multiplexer (DZ)

The DZ11 is an 8-line terminal multiplexer.  Up to 32 DZ11's (256 lines) are supported.  The default number of lines is 32.  The number of lines can be changed with the command

SET DZ LINES=n			set line count to n

The line count must be a multiple of 8, with a maximum of 256.

The DZ11 supports three character processing modes, 7P, 7B, and 8B:

mode	input characters		output characters

7P	high-order bit cleared	high-order bit cleared,

non-printing characters suppressed

7B	high-order bit cleared	high-order bit cleared

8B	no changes			no changes

The default is 8B.

The DZ11 supports logging on a per-line basis.  The command

SET DZ LOG=line=filename

enables logging for the specified line to the indicated file.  The command

SET DZ NOLOG=line

disables logging for the specified line and closes any open log file.  Finally, the command

SHOW DZ LOG

displays logging information for all DZ lines.

The terminal lines perform input and output through Telnet sessions connected to a user-specified port.  The ATTACH command specifies the port to be used:

ATTACH {-am} DZ <port>		set up listening port

where port is a decimal number between 1 and 65535 that is not being used for other TCP/IP activities.  The optional switch -m turns on the DZ11's modem controls; the optional switch -a turns on active disconnects (disconnect session if computer clears Data Terminal Ready).  Without modem control, the DZ behaves as though terminals were directly connected; disconnecting the Telnet session does not cause any operating system-visible change in line status.

Once the DZ is attached and the simulator is running, the DZ will listen for connections on the specified port.  It assumes that the incoming connections are Telnet connections.  The connection remains open until disconnected by the simulated program, the Telnet client, a SET DZ DISCONNECT command, or a DETACH DZ command.

Other special DZ commands:

SHOW DZ CONNECTIONS 		show current connections

SHOW DZ STATISTICS 		show statistics for active connections

SET DZ DISCONNECT=linenumber	disconnects the specified line.

The DZ11 implements these registers:

name			size		comments

CSR[0:3]		16		control/status register, boards 0 to 3

RBUF[0:3]		16		receive buffer, boards 0 to 3

LPR[0:3]		16		line parameter register, boards 0 to 3

TCR[0:3]		16		transmission control register, boards 0 to 3

MSR[0:3]		16		modem status register, boards 0 to 3

TDR[0:3]		16		transmit data register, boards 0 to 3

SAENB[0:3]		1		silo alarm enabled, boards 0 to 3

RXINT			4		receive interrupts, boards 3 to 0

TXINT			4		transmit interrupts, boards 3 to 0

MDMTCL		1		modem control enabled

AUTODS		1		autodisconnect enabled

The DZ11 does not support save and restore.  All open connections are lost when the simulator shuts down or the DZ is detached.

### DHU11 Terminal Multiplexer (VH)

The DHU11 is an 16-line terminal multiplexer for Unibus systems.  Up to 4 DHU11's are supported.

The number of active DHU11 devices is configured by setting the number of multiplexer lines desired.  The number of DHU11 lines can be set as follows:

SET VH LINES=n			n must be a multiple of 16.

The DHU11 is a programmable asynchronous terminal multiplexer.  The register sets are compatible with these devices.  For transmission, the DHU11 can be used in either DMA or programmed I/O mode.  For reception, there is a 256-entry FIFO for received characters, dataset status changes, and diagnostic information, and a programmable input interrupt timer.  The device supports 16-, 18-, and 22-bit addressing.  The DHU11 can be programmed to filter and/or handle XON/XOFF characters independently of the processor.  The DHU11 supports programmable bit width (between 5 and 8) for the input and output of characters.

The VH controller may be adjusted on a per controller basis as follows:

SET VHn DHU				use the DHU programming mode and registers

SET VHn DHV				use the DHV programming mode and registers

DMA output is supported.  In a real DHU11, DMA is not initiated immediately upon receipt of TX.DMA.START but is dependent upon some internal processes.  The VH controller mimics this behavior by default.  It may be desirable to alter this and start immediately, though this may not be compatible with all operating systems and diagnostics.  You can change the behavior of the VH controller as follows:

SET VHn NORMAL			use normal DMA procedures

SET VHn FASTDMA			set DMA to initiate immediately

The terminal lines perform input and output through Telnet sessions connected to a user-specified port.  The ATTACH command specifies the tcp listening port to be used:

ATTACH VH <port>			set up listening port

where port is a decimal number between 1 and 65535 that is not  being used for other TCP/IP activities.  This port is the point of entry for al lines on all VH controllers.

Modem and auto-disconnect support may be set on an individual controller basis.  The SET MODEM command directs the controller to report modem status changes to the computer.  The SET HANGUP command turns on active disconnects (disconnect session if computer clears Data Terminal Ready).

SET VHn [NO]MODEM			disable/enable modem control

SET VHn [NO]HANGUP		disable/enable disconnect on DTR drop

Once the VH is attached and the simulator is running, the VH will listen for connections on the specified port.  It assumes that the incoming connections are Telnet connections.  The connection remains open until disconnected by the simulated program, the Telnet client, a SET VH DISCONNECT command, or a DETACH VH command.

Other special VH commands:

SHOW VH CONNECTIONS 		show current connections

SHOW VH STATISTICS 		show statistics for active connections

SET VH DISCONNECT=linenumber	disconnects the specified line.

The DHU11 implements these registers, though not all can be examined from SCP:

name            size    	comments

CSR[0:3]        16      	control/status register, boards 0 to 3

RBUF[0:3]       16      	receive buffer, boards 0 to 3

LPR[0:3]        16      	line parameter register, boards 0 to 3

RXINT           4       	receive interrupts, boards 3..0

TXINT           4       	transmit interrupts, boards 3..0

The DHU11 partially supports save and restore.  Restoring a saved configuration will reestablish the listening port configuration, however all active sessions will be lost.  All open connections are lost when the simulator shuts down or the VH is detached.

### DELUA/DEUNA Unibus Ethernet Controllers (XU, XUB)

The simulator implements two DELUA/DEUNA Unibus Ethernet controllers (XU, XUB).  Initially, both XU and XQB are disabled.  Options allow control of the MAC address, the controller mode, and the sanity timer.

SET  XU MAC=<mac-address>	ex. 08-00-2B-CC-DD-EE

SHOW XU MAC

These commands are used to change or display the MAC address.  <mac-address> is a valid ethernet MAC, delimited by dashes or periods.  The controller defaults to 08-00-2B-CC-DD-EE, which should be sufficient if there is only one SIMH controller on your LAN.  Two cards with the same MAC address will see each other's packets, resulting in a serious mess.

SET  XU TYPE={DEUNA|[DELUA]}

SHOW XU TYPE

These commands are used to change or display the controller mode. DELUA mode is better and faster but may not be usable by older or non-DEC OS's.

SHOW XU STATS

This command will display the accumulated statistics for the simulated Ethernet controller.

SHOW XU FILTERS

This command will display the MAC addresses which the controller will receive packets addressed to.

To access the network, the simulated Ethernet controller must be attached

to a real Ethernet interface:

ATTACH XU {ethX|<device_name>}	ex. eth0 or /dev/era0

SHOW XU ETH

where X in 'ethX' is the number of the Ethernet controller to attach, or the real device name.  The X number is system dependent.  If you only have one Ethernet controller, the number will probably be 0. To find out what your system thinks the Ethernet numbers are, use the SHOW XQ ETH command.  The device list can be quite cryptic, depending on the host system, but is probably better than guessing. If you do not attach the device, the controller will behave as though the Ethernet cable were unplugged.

One final note: because of its asynchronous nature, the XU controller is not limited to the ~1.5Mbit/sec of the real DEUNA/DELUA controllers, nor the 10Mbit/sec of a standard Ethernet.  Attach it to a Fast Ethernet (100 Mbit/sec) card, and "Feel the Power!" :-)

### DMC11/DMR11 Unibus DDCMP Controllers

The DMC11 is a communication subsystem which consists of a microprocessor based, intelligent synchronous communications controller which employs the DIGITAL Data Communications Message Protocol (DDCMP).

#### Hardware Description

The DMC11 consists of a microprocessor module and a synchronous line unit module.

##### Models

There were a number of microprocessor controlled DDCMP devices produced.

###### DMC11

The original kmc11 microprocessor board with DMC microcode and a sync line unit.

###### DMR11

The more advanced kmc11 microprocessor board with DMR microcode and a sync line unit.

###### DMP11

A newly designed Unibus board with a more complete programming interface and a sync line unit.

###### DMV11

A Qbus version of the DMP11 with some more advanced refinements and a sync line unit.

##### Registers

The DMC device implements these registers:

| Name | Size | Description |
| --- | --- | --- |
| DMC RXINT | 32 | input interrupt summary |
| DMC TXINT | 32 | output interrupt summary |
| DMC POLL | 32 | connect poll interval |
| DMC SEL0[0:7] | 16 | Select 0 CSR |
| DMC SEL2[0:7] | 16 | Select 2 CSR |
| DMC SEL4[0:7] | 16 | Select 4 CSR |
| DMC SEL6[0:7] | 16 | Select 6 CSR |
| DMC SPEED[0:7] | 32 | line speed |
| DMC PEER[0:5119] | 8 | peer address:port |
| DMC PORT[0:5119] | 8 | listen port |
| DMC BASEADDR[0:7] | 32 | program set base address |
| DMC BASESIZE[0:7] | 16 | program set base size |
| DMC MODEM[0:7] | 8 | modem control bits |

These registers contain the emulated state of the device.  These values don't necessarily relate to any detail of the original device being emulated but are merely internal details of the emulation.

#### Configuration

A DMC device is configured with various simh SET and ATTACH commands

##### Set commands

| set DMC LINES=n | Sets the number of DMC devices being emulated |
| --- | --- |
| set DMC CONNECTPOLL=seconds | Sets the connection poll frequency |
| set DMC ADDRESS=val | Bus address |
| set DMC VECTOR=val | Interrupt vector |
| set DMC ENABLE | Enables device DMC |
| set DMC DISABLE | Disables device DMC |
| set DMC DEBUG | Enables debugging for device DMC |
| set DMC NODEBUG | Disables debugging for device DMC |
| set DMC DEBUG=TRACE;WARN;REG;INTREG;INFO;DATA;DATASUM;MODEM;CONNECT;INT |  |
|  | Enables specific debugging for device DMC |
| set DMC NODEBUG=TRACE;WARN;REG;INTREG;INFO;DATA;DATASUM;MODEM;CONNECT;INT |  |
|  | Disables specific debugging for device DMC |
| set DMCn PEER=address:port |  |
| set DMCn SPEED=bits/sec (0=unrestricted) |  |
| set DMCn TYPE={DMR,DMC} |  |
| set DMCn STATS | Display/Clear statistics |

###### Lines

A maximum of 8 DMC11 devices can be emulated concurrently in the VAX 11/780 simulator. The number of simulated DMC11 devices or lines can be specified with command:

sim> SET DMC0 LINES=n

###### Peer

To set the host and port to which data is to be transmitted use the following command:

sim> SET DMC0 PEER=host:port

###### Connectpoll

The minimum interval between attempts to connect to the other side is set using the following command:

sim> SET DMC0 CONNECTPOLL=n

Where n is the number of seconds. The default is 2 seconds.

###### Speed

If you want to experience the actual data rates of the physical hardware you can set the bit rate of the simulated line can be set using the following command:

sim> SET DMC0 SPEED=n

Where n is the number of data bits per second that the simulated line runs at.  In practice this is implemented as a delay while transmitting bytes to the socket.  Use a value of zero to run at full speed with no artificial throttling.

###### Type

The type of device being emulated can be changed with the following command:

sim> SET DMC0 TYPE={DMR,DMC}

A SET TYPE command should be entered before the device is attached to a listening port.

##### Attach

The device must be attached to a receive port, use the ATTACH command specifying the receive port number.

sim> ATTACH DMC0 port

The Peer host:port value must be specified before the attach command.

##### Examples

To configure two simulators to talk to each other use the following example:

Machine 1

sim> SET DMC ENABLE

sim> SET DMC0 PEER=LOCALHOST:2222

sim> ATTACH DMC0 1111

Machine 2

sim> SET DMC ENABLE

sim> SET DMC0 PEER=LOCALHOST:1111

sim> ATTACH DMC0 2222

#### Monitoring

The DMC device and DMC0 line configuration and state can be displayed with one of the available show commands.

##### Show commands

| show DMC LINES | Display number of devices |
| --- | --- |
| show DMC CONNECTPOLL | Display connection poll interval |
| show DMC ADDRESS | Bus address |
| show DMC VECTOR | Interrupt vector |
| show DMC DEBUG | Displays debugging status for device DMC |
| show DMCn PEER | Display destination/source |
| show DMCn SPEED | Display rate limit |
| show DMCn TYPE | Set/Display device type |
| show DMCn STATS | Display/Clear statistics |
| show DMCn QUEUES | Display Queue state |
| show DMCn DDCMP | Display DDCMP state information |

#### Restrictions

Real hardware synchronous connections could operate in Multi-Point mode.  Multi-Point mode was a way of sharing a single wire with multiple destination systems or devices.  Multi-Point mode is not currently emulated by this or other simulated synchronous devices.

In real hardware, the DMC11 spoke a version of DDCMP which peer devices needed to be aware of.  The DMR11, DMP11, and DMV11 boards have configuration switches or programatic methods to indicate that the peer device was a DMC11.  The emulated devices all speak the same level of DDCMP so no special remote device awareness need be considered.

#### Implementation

A real DMC11 transports data using DDCMP via a synchronous connection, the emulated device makes a TCP/IP connection to another emulated device which either speaks DDCMP over the TCP connection directly, or interfaces to a simulated computer where the operating system speaks the DDCMP protocol on the wire.

The DMC11 can be used for point-to-point DDCMP connections carrying DECnet and other types of networking, e.g. from ULTRIX or DSM.

#### Debugging

The simulator has a number of debug options, these are:

| REG | Shows whenever a CSR is programatically read or written and the current value. |
| --- | --- |
| INTREG | Shows internal register value changes. |
| INFO | Shows higher-level tracing only. |
| WARN | Shows any warnings. |
| TRACE | Shows more detailed trace information. |
| DATASUM | Brief summary of each received and transmitted buffer. Ignored if DATA is set. |
| DATA | Shows the actual data sent and received. |
| MODEM | Shows modem signal transitions details. |
| CONNECT | Shows sockets connection activity. |
| INT | Shows Interrupt activity. |

To get a full trace use

sim> SET DMC DEBUG

However it is recommended to use the following when sending traces:

sim> SET DMC DEBUG=REG;INFO;WARN

#### Related Devices

The DMC can facilitate communication with other simh simulators which have emulated synchronous network devices available.  These include the following:

DUP11*       Unibus PDP11 simulators

DPV11*       Qbus PDP11 simulators

KDP11*       Unibus PDP11 simulators and PDP10 simulators

DMR11        Unibus PDP11 simulators and Unibus VAX simulators

DMC11        Unibus PDP11 simulators and Unibus VAX simulators

DMP11        Unibus PDP11 simulators and Unibus VAX simulators

DMV11        Qbus VAX simulators

- Indicates systems which have OS provided DDCMP implementations.The DMC11 is a synchronous serial point-to-point communications device. A real DMC11 transports data using DDCMP, the emulated device makes a TCP/IP connection to another emulated device and sends length-prefixed messages across the connection, each message representing a single buffer passed to the DMC11. The DMC11 can be used for point-to-point DDCMP connections carrying DECnet and other types of networking, e.g. from ULTRIX or DSM.

## CR11 Card Reader (CR)

The card reader (CR) implements a single controller (the CR11) and card reader (e.g., Documation M200, GDI Model 100) by reading a file and presenting lines or cards to the simulator.  Card decks may be represented by plain text ASCII files, card image files, or column binary files.  The CR11 controller is also compatible with the CM11-F, CME11, and CMS11.

Card image files are a file format designed by Douglas W. Jones at the University of Iowa to support the interchange of card deck data.  These files have a much richer information carrying capacity than plain ASCII files.  Card Image files can contain such interchange information as card-stock color, corner cuts, special artwork, as well as the binary punch data representing all 12 columns.  Complete details on the format, as well as sample code, are available at Prof. Jones's site: http://www.cs.uiowa.edu/~jones/cards/.

Examples of the CR11 include the M8290 and M8291 (CMS11).  All card readers use a common vector at 0230 and CSR at 177160.  Even though the CR11 is normally configured as a BR6 device, it is configured for BR4 in this simulation.

The card reader supports ASCII, card image, and column binary format card “decks.”  When reading plain ASCII files, lines longer than 80 characters are silently truncated.  Card image support is included for 80 column Hollerith, 82 column Hollerith (silently ignoring columns 0 and 81), and 40 column Hollerith (mark-sense) cards.  Column binary supports 80 column card images only.  All files are attached read-only (as if the -R switch were given).

ATTACH –A CR <file>			file is ASCII text

ATTACH –B CR <file>			file is column binary

ATTACH –I CR <file>			file is card image format

If no flags are given, the file extension is evaluated.  If the filename ends in .TXT, the file is treated as ASCII text.  If the filename ends in .CBN, the file is treated as column binary.  Otherwise, the CR driver looks for a card image header.  If a correct header is found the file is treated as card image format, otherwise it is treated as ASCII text.

The correct character translation MUST be set if a plain text file is to be used for card deck input.  The correct translation SHOULD be set to allow correct ASCII debugging of a card image or column binary input deck.  Depending upon the operating system in use, how it was generated, and how the card data will be read and used, the translation must be set correctly so that the proper character set is used by the driver.  Use the following command to explicitly set the correct translation:

SET TRANSLATION={DEFAULT|026|026FTN|029|EBCDIC}

This command should be given after a deck is attached to the simulator.  The mappings above are completely described at http://www.cs.uiowa.edu/~jones/cards/codes.html.  Note that DEC typically used 029 or 026FTN mappings.

DEC operating systems used a variety of methods to determine the end of a deck (recognizing that 'hopper empty' does not necessarily mean the end of a deck.  Below is a summary of the various operating system conventions for signaling end of deck:

RT-11:	12-11-0-1-6-7-8-9 punch in column 1

RSTS/E:	12-11-0-1 or 12-11-0-1-6-7-8-9 punch in column 1

RSX:		12-11-0-1-6-7-8-9 punch

VMS:		12-11-0-1-6-7-8-9 punch in first 8 columns

TOPS:		12-11-0-1 or 12-11-0-1-6-7-8-9 punch in column 1

Using the AUTOEOF setting, the card reader can be set to automatically generate an EOF card consisting of the 12-11-0-1-6-7-8-9 punch in columns 1-8.  When set to CD11 mode, this switch also enables automatic setting of the EOF bit in the controller after the EOF card has been processed.  [The CR11 does not have a similar capability.]  By default AUTOEOF is enabled.

SET CR AUTOEOF

SET CR NOAUTOEOF

The default card reader rate for the CR11 is 285 cpm.  The reader rate can be set to its default value or to anywhere in the range 200 to 1200 cpm.  This rate may be changed while the unit is attached.

SET CR RATE={DEFAULT|200 to 1200}

It is standard operating procedure for operators to load a card deck and press the momentary action RESET button to clear any error conditions and alert the processor that a deck is available to read.  Use the following command to simulate pressing the card reader RESET button,

SET CR RESET

Another common control of physical card readers is the STOP button.  An operator could use this button to finish the read operation for the current card and terminate reading a deck early.  Use the following command to simulate pressing the card reader STOP button.

SET CR STOP

The simulator does not support the BOOT command.  The simulator does not stop on file I/O errors.  Instead the controller signals a reader check to the CPU.

The CR controller implements these registers:

name			size		comments

BUF			8		ASCII value of last column processed

CRS			16		CR11 status register

CRB1			16		CR11 12-bit Hollerith character

CRB2			16		CR11 8-bit compressed character

CRM			16		CR11 maintenance register

CDST			16		CD11 control/status register

CDCC			16		CD11 column count

CDBA			16		CD11 current bus address

CDDB			16		CD11 data buffer, 2nd status

BLOWER		2		blower state value

INT			1		interrupt pending flag

ERR			1		error flag (CRS<15>)

IE			1		interrupt enable flag (CRS<6>)

POS			32		file position - do not alter

TIME			24		delay time between columns

# Symbolic Display and Input

The VAX simulator implements symbolic display and input.  Display is controlled by command line switches:

-a,-c			display as ASCII data

-m			display instruction mnemonics

-p			display compatibility mode mnemonics

-r			display RADIX50 encoding

Input parsing is controlled by the first character typed in or by command line switches:

' or -a		ASCII characters (determined by length)

" or -c		ASCII string (maximum 60 characters)

-p			compatibility mode instruction mnemonic

alphabetic		instruction mnemonic

numeric		octal number

VAX instruction input uses standard VAX assembler syntax.  Compatibility mode instruction input uses standard PDP-11 assembler syntax.

The syntax for VAX specifiers is as follows:

syntax		specifier	displacement	comments

#s^n, #n		0n		-		short literal, integer only

[Rn]			4n		-		indexed, second specifier follows

Rn			5n		-		PC illegal

(Rn)			6n		-		PC illegal

-(Rn)			7n          -		PC illegal

(Rn)+			8n		-

#i^n, #n		8F		n		immediate

@(Rn)+		9n		-

@#addr		9F		addr		absolute

{+/-}b^d(Rn)	An		{+/-}d	byte displacement

b^d			AF		d - PC	byte PC relative

@{+/-}b^d(Rn)	Bn		{+/-}d	byte displacement deferred

@b^d			BF		d - PC	byte PC relative deferred

{+/-}w^d(Rn)	Cn		{+/-}d	word displacement

w^d			CF		d - PC	word PC relative

@{+/-}w^d(Rn)	Dn		{+/-}d	word displacement deferred

@w^d			DF		d - PC	word PC relative deferred

{+/-}l^d(Rn)	En		{+/-}d	long displacement

l^d			EF		d - PC	long PC relative

@{+/-}l^d(Rn)	Fn		{+/-}d	long displacement deferred

@l^d			FF		d - PC	long PC relative deferred

If no override is given for a literal (s^ or i^) or for a displacement or PC relative address (b^, w^, or l^), the simulator chooses the mode automatically.