# Source-Table

Extract from http://igs.ifag.de/root_ftp/NTRIP/documentation/NtripDocumentation.pdf

The NtripCaster maintains a source-table containing information on available NtripSources, networks of NtripSources, and NtripCasters, to be sent to an NtripClient on request. Note that to request a source-table from the NtripCaster, the NtripClient uses the client message (see Fig. 3) while leaving out the mountpoint parameter.

Source-table records are dedicated to one of the following:

- Data STReams (record type STR),

- CASters (record type CAS), or

- NETworks of data streams (record type NET).

This structure is expandable by introducing new record types when necessary. Older NtripClient versions might ignore newly introduced record types. All NtripClients must be able to decode record type STR. Decoding types CAS and NET is an optional feature.

All data fields in the source-table records are separated using the semicolon character “;” as a field delimiter. In case the semicolon character becomes part of the content of a data field, it must be quoted: “;”. The number of data fields in the source-table records is not fixed. Introducing additional data fields is allowed. The last data field always contains “Miscellaneous” information.

The source-table is initiated by the HTTP/1.1 header fields

Server: <NtripCasterIdentifier>/<NtripVersion><CR><LF>
Content-Type: text/plain<CR><LF>
Content-Length: <Content-Length><CR><LF>
<CR><LF>

followed by the actual source-table records. The “Server:” record contains

- Before the slash: an NtripCaster identifier to be defined by the NtripCaster operator (see parameter #4 in Table 2)

- After the slash: an Ntrip version number (e.g. NtripV1.0 or 1.0) in order to allow an NtripClient to understand which version of Ntrip is supported by a particular NtripCaster.

The content-length indicates the size of the source-table records (decimal number of octets, e.g. “Content-Length: 243”).

The end of the source-table is notified by the string: ENDSOURCETABLE

Table 1: Format and Contents of Source-Table Records Describing a Data Stream

| # | Record Parameter | Meaning | Format | Examples |
| --- | --- | --- | --- | --- |
| 1 | <type> = STR | The following parameters of this record describe a data stream | 3 Characters | STR (the only acceptable string) |
| 2 | <mountpoint> | Caster mountpoint | Characters, 100 max. | LEIJ0 LEIJ1 WTZJ0 |
| 3 | <identifier> | Source identifier, e.g. name of city next to source location | Characters, undefined length | Frankfurt |
| 4 | <format> | Data format RTCM, RAW, etc. | Characters, undefined length | RTCM 2.0 RTCM 2.1 RTCM 2.3 CMR CMR+ RTCM 2 RAW RTCA |
| 5 | <format-details> | E.g. RTCM message types or RAW data format etc., update periods in parenthesis in seconds | Characters, undefined length | 1(1), 2(1), 3(30) MBEN(1) LB2 |
| 6 | <carrier> | Data stream contains carrier phase information 0 = No (e.g. for DGPS) 1 = Yes, L1 (e.g. for RTK) 2 = Yes, L1&L2 (e.g. for RTK) | Integer | 0 1 2 |
| 7 | <nav-system> | Navigation system(s) | Characters, undefined length | GPS GPS+GLONASS GPS+EGNOS |
| 8 | <network> | Network | Characters, undefined length | EUREF IGS IGLOS SAPOS GREF Misc |
| 9 | <country> | Three character country code in ISO 3166 | 3 Characters | DEU ITA ESP |
| 10 | <latitude> | Position, latitude, north (approximate position in case of nmea = 1) | Floating point number, two digits after decimal point | 40.12 -12.14 |
| 11 | <longitude> | Position, longitude, east (approximate position in case of nmea = 1) | Floating point number, two digits after decimal point | 10.12 357.85 |
| 12 | <nmea> | Necessity for Client to send NMEA message with approximate position to Caster 0 = Client must not send NMEA message with approximate position to Caster 1 = Client must send NMEA GGA message with approximate position to Caster | Integer | 0 1 |
| 13 | <solution> | Stream generated from single reference station or from networked reference stations 0 = Single base 1 = Network | Integer | 0 1 |
| 14 | <generator> | Hard- or software generating data stream | Characters, undefined length | JPS Legacy E GPSNet |
| 15 | <compr-encryp> | Compression/Encryption algorithm applied | Characters, undefined length | none |
| 16 | <authentication> | Access protection for this particular data stream N = None B = Basic D = Digest | 1 Character | N B D |
| 17 | <fee> | User fee for receiving this particular data stream N = No user fee Y = Usage is charged | 1 Character | N Y |
| 18 | <bitrate> | Bit rate of data stream, bits per second | Integer | 500 5000 |
| … | … |  |  |  |
| … | … |  |  |  |
| n | <misc> | Miscellaneous information, last data field in record | Characters, undefined length | none Demo |

Table 2: Format and Contents of Source-Table Records Describing a Caster

| # | Source-table Element | Meaning | Format | Examples |
| --- | --- | --- | --- | --- |
| 1 | <type> = CAS | The following parameters of this record describe a Caster | 3 Characters | CAS (the only acceptable string) |
| 2 | <host> | Caster Internet host domain name or IP address | Characters, 128 max. | 141.74.243.11 euref-ip.ifag.de |
| 3 | <port> | Port number | Integer | 80 2101 |
| 4 | <identifier> | Caster identifier, e.g. name of provider | Characters, undefined length | NTRIP Caster/0.5.3 Trimble-iGate |
| 5 | <operator> | Name of institution / agency / company operating the Caster | Characters, undefined length | BKG Geo++ |
| 6 | <nmea> | Capability of Caster to receive NMEA message with approximate position from Client 0 = Caster is not able to handle incoming NMEA message with approximate position from Client 1 = Caster is able to handle incoming NMEA GGA message with approximate position from Client | Integer | 0 1 |
| 7 | <country> | Three character country code in ISO 3166 | 3 Characters | DEU ITA ESP |
| 8 | <latitude> | Position, latitude, north | Floating point number, two digits after decimal point | 40.12 -12.14 |
| 9 | <longitude> | Position, longitude, east | Floating point number, two digits after decimal point | 10.12 357.85 |
| 10 | <fallback_host> | Fallback Caster IP address No Fallback: 0.0.0.0 | Characters, 128 max. | 213.20.169.236 caster.fgi.fi 0.0.0.0 |
| 11 | <fallback_port> | Fallback Caster port number No Fallback: 0 | Integer | 80 2101 0 |
| … |  |  |  |  |
| … |  |  |  |  |
| n | <misc> | Miscellaneous information, last data field in record | Characters, undefined length | none |

Table 3: Format and Contents of Source-Table Records Describing a Network of Data Streams

| # | Source-table Element | Meaning | Format | Examples |
| --- | --- | --- | --- | --- |
| 1 | <type> = NET | The following parameters of this record describe a network of data streams | 3 Characters | NET (the only acceptable string) |
| 2 | <identifier> | Network identifier, e.g. name of a network of GNSS permanent reference stations | Characters, undefined length | EPN IGLOS GREF SAPOS-THR |
| 3 | <operator> | Name of institution / agency / company operating the network | Characters, undefined length | EUREF IGS TRIMBLE ASCOS GEO++ SAPOS-THR |
| 4 | <authentication> | Access protection for data streams of the network N = None B = Basic D = Digest | 1 Character | N B D N,B |
| 5 | <fee> | User fee for receiving data streams from this network N = No user fee Y = Usage is charged | 1 Character | N Y N,Y |
| 6 | <web-net> | Web-address for network information | Characters, undefined length | http://igs.ifag.de |
| 7 | <web-str> | Web-address for stream information | Characters, undefined length | http://www.epncb.oma.be none |
| 8 | <web-reg> | Web address or mail address for registration | Characters, undefined length | euref-ip@ifag.de http://igs.ifag.de |
| … |  |  |  |  |
| … |  |  |  |  |
| n | <misc> | Miscellaneous information, last data field in record | Characters, undefined length | none |

Example Source-Table

SOURCETABLE 200 OK

Server :  NTRIP Caster 1.5.5/1.0
Date : 23/Jan/2004:08:54:59 UTC

Content-Type: text/plain

Content-Length: 6999

CAS;129.217.182.51;80;EUREF;BKG;0;DEU;51.5;7.5;http://igs.ifag.de/index_ntrip_cast.htm

CAS;62.159.109.248;8080;Trimble GPSNet;Trimble Terrasat;1;DEU;48.03;11.72;http://www.virtualrtk.com

NET;EUREF;EUREF;B;N;http://www.epncb.oma.be/euref_IP;http://www.epncb.oma.be/euref_IP;http://www.epncb.oma.be/projects/euref_IP/NtripRegister.html;none

NET;IGS;BKG;B;N;http://igscb.jpl.nasa.gov/;none;http://igs.ifag.de/root_ftp/software/NtripRegister.doc;none

NET;GREF;BKG;B;N;http://igs.ifag.de/gref_realtime.htm;http://igs.ifag.de/root_ftp/misc/ntrip/streamlist.htm;http://igs.ifag.de/root_ftp/software/NtripRegister.doc;none

NET;ascos;Ruhrgas AG;B;N;http://www.ascos.de;none;http://igs.ifag.de/root_ftp/software/NtripRegister.doc;none

NET;SCIGN-OCRT;SOPAC,CSRC;B;N;http://www.scign.org;none;ybock@ucsd.edu;none

NET;CORS;NGS;B;N;http://www.ngs.noaa.gov/CORS;http://igs.ifag.de/root_ftp/misc/ntrip/streamlist.htm;http://igs.ifag.de/root_ftp/software/NtripRegister.doc;none

NET;Misc;BKG;B;N;http://igs.ifag.de/root_ftp/misc/ntrip/streamlist.htm;none;http://igs.ifag.de/root_ftp/software/NtripRegister.doc;none

NET;EGNOS;BKG;B;N;http://igs.ifag.de/root_ftp/misc/ntrip/streamlist.htm;none;http://igs.ifag.de/root_ftp/software/NtripRegister.doc;none

NET;OmniSTAR;BKG;B;Y;http://www.omnistar.nl;http://www.omnistar.nl/graphs.asp;http://www.omnistar.nl/purchase.asp;Test and Evaluation only

STR;FFMJ2;Frankfurt;RTCM 2.1;1(1),3(19),16(59);0;GPS;GREF;DEU;50.12;8.68;0;1;GPSNet V2.10;none;N;N;560;Demo

STR;FFMJ1;Frankfurt;RTCM 2.2;3(19),16(59),18(1),19(1);2;GPS;GREF;DEU;50.09;8.66;0;0;GPSNet V1.9;none;N;N;2800;Demo

STR;FFMT0;Frankfurt;RTCM 2.0;1(3),2(90),3(90),16(90);0;GPS;Misc;DEU;50.09;8.66;0;0;Trimble 4000SSI;none;N;N;190;ALF Demo

STR;CONZ0;Concepcion-TIGO;RAW;Compact(1);2;GPS+GLO;IGS;CHL;-36.84;286.98;0;0;Javad Legacy E;none;B;N;3600;none

STR;FFMJ0;Frankfurt;RAW;Compact(1);2;GPS+GLO;IGS;DEU;50.09;8.66;0;0;Javad Legacy E;none;N;N;3600;Demo

STR;LEIJ0;Leipzig;RAW;Compact(1);2;GPS+GLO;IGS;DEU;51.33;12.37;0;0;Javad Legacy E;none;B;N;3600;none

STR;LDBG0;Lindenberg;RAW;Compact(1);2;GPS+GLO;IGS;DEU;52.21;14.12;0;0;Javad Legacy E;none;B;N;3600;none

STR;MOXA0;Moxa;RAW;Compact(1);2;GPS+GLO;IGS;DEU;50.64;11.61;0;0;Javad Legacy E;none;B;N;3600;none

STR;MUEJ0;Muenchen;RAW;Compact(1);2;GPS+GLO;IGS;DEU;48.13;11.57;0;0;Javad Legacy E;none;B;N;3600;none

STR;WARN0;Warnemuende;RAW;Compact(1);2;GPS+GLO;IGS;DEU;54.17;12.10;0;0;Javad Legacy E;none;B;N;3600;none

STR;BARC0;Barcelona;RTCM 2.0;1(1),3(3),16(90);0;GPS;Misc;ESP;41.37;2.16;0;0;Novatel OEM3;none;B;N;230;ICC, Geod. Dep.

STR;BOCH0;Bochum;RTCM 2.1;1(1)3,16,18(1),19(1);2;GPS;Misc;DEU;50.45;7.27;0;0;none;none;B;N;13000;FH Bochum

STR;BRUS0;Brussels;RTCM 2.0;1(1),3(60),16;0;GPS;Misc;BEL;50.80;4.36;0;0;Ashtech UZ-12;none;B;N;500;ROB

STR;SP300;Ephemerides;SP3;ASCII, last 8 15min Epochs;0;GPS;Misc;FIN;60.10;24.50;0;1;IGS Ultra Rapid Orbit;none;B;N;5600;Finnish Geodetic Institute

STR;BUCU0;Bucharest;RTCM 2.0;1(1),3(60),16(60);0;GPS;EUREF;ROU;44.46;26.12;0;0;Ashtech Z-XII3;none;B;N;520;TU Bucharest

STR;CAGZ0;Cagliari;RTCM 2.1;1(3),3(60),16(60),18(1),19(1),31(3);2;GPS+GLO;EUREF;ITA;39.14;8.97;0;0;JPS Eurocard;none;B;N;3900;Univ Cagliari, DIST

STR;STEI0;Steinbrink;RAW;Compact(1);2;GPS+GLO;ascos;DEU;52.50;8.73;0;0;Javad Legacy E;none;B;N;3600;ALLSAT

STR;UMME0;Ummeln;RAW;Compact(1);2;GPS+GLO;ascos;DEU;51.98;8.46;0;0;Javad Legacy E;none;B;N;3600;ALLSAT

STR;HELS0;Helsinki;RTCA;Thales ASCII;1;EGNOS;Misc;FIN;60.10;24.50;0;0;Thales DG-16;none;B;N;700;Finnish Geodetic Institute

STR;HELS1;Helsinki;RTCM 3.0;1,2,3,6,9,16;0;GPS,EGNOS;Misc;FIN;60.10;24.50;0;0;Thales DG-16;none;B;N;700;Finnish Geodetic Institute

STR;HERT0;Hailsham;RTCM 2.2;1(1),3(60),18(1),19(1),22(60),31(1);2;GPS+GLO;EUREF;GBR;50.87;0.33;0;0;Ashtech Z18;none;B;N;5200;NERC

STR;KRAW0;Krakow;RTCM 2.2;1(1),3(60),16(60),18(1),19(1),22(60);2;GPS;EUREF;POL;50.01;19.92;0;0;Ashtech UZ-12;none;B;N;1900;AGH

STR;KOPE0;Koper;RTCM 2.1;1(5),3(69),20(1),21(1),22;2;GPS;Misc;SLO;48.40;17.03;0;0;none;none;B;N;2500;Harpha Sea

STR;GAVL0;Gavle;RTCM 2.3;1(1),3(19),16(59);0;GPS+GLO;Misc;SWE;60.67;17.14;0;0;Javad Legacy;none;B;N;800;National Land Survey Sweden

STR;MILU0;Milano;RTCM 2.1;1(1),3(60),16(60),18(1),19(1);2;GPS;Misc;ITA;45.48;9.23;0;0;Ashtech Z12;none;B;N;2800;University Milano, Agricultural Institute

STR;MADR0;Madrid;RTCM 2.1;1(1),3(10),16,18(1),19(1);2;GPS;Misc;ESP;40.44;356.29;0;0;Ashtech ZSensor;none;B;N;3300;IGNE, Serv. Geod.

STR;MALA0;Malaga;RTCM 2.0;1(1),2(1),3(30);0;GPS;Misc;ESP;36.73;355.61;0;0;Trimble 4000SSI;none;B;N;1200;IGNE, Serv. Geod.

STR;OBET0;Oberpfaffenhofen;RAW;MBEN/PBEN/SNAV/SALM(1);2;GPS;Misc;DEU;48.08;11.28;0;0;Ashtech Z12;none;B;N;8000;DLR

STR;PADO0;Padova;RTCM 2.1;1(1),3(10),16(30),18(1),19(1),59;2;GPS;IGS;ITA;45.41;11.90;0;0;Trimble 4000SSI;none;B;N;3600;Univ Padova

STR;MAT10;Matera;RTCM 2.1;3(60),16(60),18(1),19(1),22(60),36(1);2;GPS+GLO;IGS;ITA;40.65;16.70;0;0;Ashtech Z18;none;B;N;5600;Agenzia Spaziale Italiana

STR;GOPE0;Praha-Ondrejov;RTCM 2.2;1(1),3(60),16(60),18(1),19(1),22(60),31(1),36(1);2;GPS+GLO;EUREF;CZE;49.91;14.79;0;0;Ashtech Z18;none;B;N;5600;Uni Praha

STR;ROMA0;Roma;RTCM 2.1;1(1),3(10),18(1),19(1),59;2;GPS;Misc;ITA;42.00;12.50;0;0;Trimble 4000;none;B;N;3400;Univ Roma

STR;ROMG0;Roma;RTCM 2.1;9(1),34(1);0;GPS;Misc;ITA;42.00;12.50;0;0;Trimble 5700;none;B;N;500;Galileo Sistemi

STR;SYDN0;Sydney;RTCM 2.1;3(30),18(1),19(1);2;GPS;Misc;AUS;-33.92;151.23;0;0;Leica MC500;none;B;N;3200;University of New South Wales

STR;TORI0;Torino;RTCM 2.1;1(1),3(60),16(30),18(1),19(1);2;GPS;Misc;ITA;45.06;7.66;0;0;none;none;B;N;12000;Politecnico di Torino

STR;PENC0;Penc-RT;RTCM 2.0;1(1),3(10),22,59;0;GPS;EUREF;HUN;47.79;19.28;0;0;Trimble 5700;none;B;N;700;FOMI, SGO

STR;SZEK0;Szekesfehervar;RTCM 2.0;1(1),3(10);0;GPS;Misc;HUN;47.18;19.42;0;0;GPSBase;none;B;N;700;FOMI, SGO

STR;BERJ0;Berlin;RTCM 2.1;1(1),3(19),16(59);0;GPS;GREF;DEU;52.52;13.38;0;1;GPSNet V2.10;none;B;N;560;VRS

STR;DREJ1;Dresden;RTCM 2.1;1(1),3(19),16(59);0;GPS;GREF;DEU;51.05;13.73;0;1;GPSNet V2.10;none;B;N;560;VRS

STR;WILH0;Wilhelmshaven;RTCM 2.1;1(1),3(19),16(59);0;GPS;GREF;DEU;53.52;8.10;0;1;GPSNet V2.01;none;B;N;560;VRS

STR;LEID0;Leidschendam;RTCM 2.0;1(1);0;GPS;OmniSTAR;NLD;52.10;4.41;0;1;OmniSTAR;none;B;Y;120;EA-SAT Geostationary L-band Broadcast

STR;ERLA0;Erlanger;RTCM 2.2;3(60),18(1),19(1),22(60);2;GPS;CORS;USA;39.01;275.24;0;0;Ashtech Z-XII3;none;B;N;3000;http://ncad.net

STR;SBCC0;Mission-Viejo;RTCM 2.1;3(15),18(1),19(1),22(15),59(1);2;GPS;SCIGN-OCRT;USA-CA;33.55;242.34;0;0;Ashtech Z-XII3;none;B;N;4000;none

STR;SACY0;Santa-Ana;RTCM 2.1;3(15),18(1),19(1),22(15),59(1);2;GPS;SCIGN-OCRT;USA-CA;33.74;242.10;0;0;Ashtech Z-XII3;none;B;N;4000;none

STR;SBCC1;Mission-Viejo;RAW;MBEN(1);2;GPS;SCIGN-OCRT;USA-CA;33.55;242.34;0;0;Ashtech Z-XII3;none;B;N;7300;none

STR;SACY1;Santa-Ana;RAW;MBEN(1);2;GPS;SCIGN-OCRT;USA-CA;33.74;242.10;0;0;Ashtech Z-XII3;none;B;N;7300;none
STR;TIGO0;TIGO-TEST;RAW;PBEN(5),MBEN(5);2;GPS;Test;CHL;-36.84;286.98;0;0;Ashtech Z12;none;B;N;1500;Softwaretest
ENDSOURCETABLE